RECORD OF EXPERIMENTS

Micro Processors & Embedded Systems

Submitted By

Abhishek Sharma

Enrolment No.: R171218005

SAP ID: 500067644

Semester VI

B. Tech. (Computer Sc. and Engineering)

Specialization in DevOps

School of Computer Science

UNIVERSITY OF PETROLEUM AND ENERGY STUDIES

Dehradun-248007 2020-21

EXPERIMENT 1. (a): Addition of Two Numbers

Aim: Write a program using 8085 & Verify for:

- Addition of Two 8-Bit Numbers.

- Addition of Two 16-Bit Numbers (With Carry).

Apparatus: Microprocessor Kit 8085, Key Board, Op-Code Sheet.

Theory:

- ORG Address (ORIGIN) Directive reserves the starting address for Program Code or data in specified memory array,

- LXI H (LOAD REGISTER PAIR IMMEDIATELY) loads 16-bit data in register pair designated by operand.

- LHLD Addr. (LOAD HL PAIR DIRECT) loads 16-bit data from specified address to designate in register pair.

- MOV A, M (MOVE M TO A) copies the data byte into accumulator from the memory specified by the address in H-L pair.

- MVI (MOVE IMMEDIATE DATA) moves immediate value to specified register.

- DAD (DOUBLE ADDITION) Add specified register pair content to HL pair content and store results into HL pair.

- JNC Addr. (JUMP IF NO CARRY) Instruction jump the execution to the specified Address if carry flag is reset.

- INR R (INCREMENT REGISTER) increment the specified register content by 1.

- INX H (INCREMENT REGISTER PAIR) increments the contents of the register pair by one.

- ADD M (ADDTION) adds the contents of memory to accumulator.

- STA addr. (STORE ACCUMULATOR DIRECT) copies the contents of the accumulator to the memory location specified in the instruction

- SHLD Addr. (STORE HL DIRECT) instruction store HL pair content to specified address.

- RST 1 (RESET) finishes the execution of the current instruction and stops any further execution.

- DB (DEFINE BYTE) Directive is defined to store values in specified memory array.

Flowchart:

For Addition Of Two 8-Bit Numbers			      For Addition Of Two 16-Bit Numbers

Program:

- For Addition of Two 8-Bit Numbers

# ORG 7000H

LXI H,7501                 // Get address of 1st no. in HL pair

MOV AM                 // Move no. into accumulator

INX H                 // HL points the address 7502 H

ADD M                 // Add the 2nd no.

INX   H                 // HL points 7503 H

MOV MA               // Store result in 7503 H

RST 1                              // Terminate

# ORG 7501H                        // Store input at the address

# DB 12H, 13H                    // Get two 8-bit no. in successive location

A. Input: 7501- 13H, 7502- 12H; Output: A- 25H, 7503- 25H

Result:

B. For Addition of Two 16-Bit Numbers

# ORG 7000H

LHLD  7601        //Get 1st no. in HL pair from memory 7601 H

XCHG              //Exchange cont. of DE       HL

LHLD  7603        //Get 2st no. in HL pair from location 7603 H

MVI C,00        //Clear reg. C.

DAD D         //Get HL+DE & store result in HL

JNC down     //If no carry move to loop/if carry then move to next step.

INR C         //Increment reg.C

MOV A, C       //Move carry from reg. C to reg. A

STA 7502        //Store carry at 7502 H

down: SHLD 7500            //Store result in 7500 H.

RST 1             //Terminate

#ORG 7601H                // Store input at the address

#DB 13,31,12,10         // Get two 16-bit no. in successive location

B. Input: 7601- 13H, 7602- 31H, 7603- 12H, 7604- 10H; Output: 7500- 25H, 7501- 41H, 7502- 00H

Result:

Precautions:

- All steps should be followed carefully.

- Make sure all power sources are disconnected.

- Make sure you are properly grounded.

- Don’t touch the live wire.

EXPERIMENT 1. (b): Subtraction of Two Numbers

Aim: Write a program using 8085 & Verify for:

- Subtraction of Two 8-Bit Numbers. (Display of Borrow)

- Subtraction of Two 16-Bit Numbers (Display of Borrow Carry).

Apparatus: Microprocessor Kit 8085 , Key Board, Op-Code Sheet.

Theory:

- ORG Addr. Directive reserves the starting address for Program Code or data in specified memory array,

- LXI H (LOAD REGISTER PAIR IMMEDIATELY) loads 16-bit data in register pair designated by operand.

- LHLD Addr. (LOAD HL PAIR DIRECT) loads 16-bit data from specified address to designate in register pair.

- MOV A, M copies the data byte into accumulator from the memory specified by the address in H-L pair.

- MVI (MOVE IMMEDIATE DATA) moves immediate value to specified register.

- SBB instruction subtracts specified register content and carry flag to Accumulator and store results into Accumulator.

- JNC Addr. Instruction jump the execution to the specified Address if carry flag is reset.

- INR instruction increment the specified register content by 1.

- INX H (INCREMENT REGISTER PAIR) increments the contents of the register pair by one.

- SUB M (SUBTRACTION) subtracts the contents of register to accumulator.

- STA address (STORE ACCUMULATOR DIRECT) copies the contents of the accumulator to the memory location specified in the instruction

- SHLD Addr. (STORE HL DIRECT) instruction store HL pair content to specified address.

- RST 1 (RESTART) finishes the execution of the current instruction and stops any further execution.

Flowchart:

For Subtraction Of Two 8-Bit Numbers		 For Subtraction Of Two 16-Bit Numbers

Program:

- For Subtraction Of Two 8-Bit Numbers

# ORG 7000H LXI H, 7501       // Get address of ist no. in HL pair

MOV A, M        // Move no. into accumulator

INX H       // HL points 7502 H.

SBB M       // Substract 2nd no. from Ist no.

INX H       //HL points 7503 H.

MOV M, A        // Move contents of acc. to memory

RST 1                      // Terminate

#ORG 7501H          // Store no. at address

#DB 20,10       // Get the two 8 bit no. at successive location

A. Input: 7501- 20H, 7502- 10H ; Output: 7503- 10H

Result:

B. For Subtraction Of Two 16-Bit Numbers

# ORG 7000H  LHLD  7501        // Get 1st 16 bit no. in HL pair

XCHG            // Exchange HL pair with DE.

LHLD  7503        // Get 2nd 16 bit no. in HL pair

MOV A, E        // Get lower byte of ist no.

SUB L         // Subtract lower byte of 2nd no.

MOV L, A        // Store the result in reg. L

MOV A, D        // Get higher byte of Ist no.

SBB H         // Subtract higher byte of 2nd no. with borrow

MOV H,A       // Move from acc. To H

SHLD  7505        // Store 16 bit result at 7505 H &7506 H

RST 1                       // Terminate

# ORG 7501H               // Store inputs at the address

# DB 30,40,10,20          // Get two 16 bit no. from successive locations

B. Input: 7501- 30H, 7502- 40H, 7503- 10H, 7504- 20H ; Output: 7505- 20H, 7506- 20H

Result:

Precautions:

- All steps should be followed carefully.

- Make sure all power sources are disconnected.

- Make sure you are properly grounded.

- Don’t touch the live wire.

EXPERIMENT 1. (c): Multiplication Of Two 8-Bit Numbers

Aim: Write a program using 8085 & Verify for: Multiplication of Two 8-Bit Numbers

Apparatus: Microprocessor Kit 8085 , Key Board, Op-Code Sheet.

Theory:

- This program multiplies two operands stored in memory location 3000H and 3001H, using successive addition method.

- In successive addition method, the second operand is considered as the counter, and the first number is added to itself until counter decrements to zero.

- Let us assume that the operands stored at memory location 0100H are 03H and 0101H is 04H. Then, by using successive addition method, we get 03H + 03H + 03H + 03H = 0CH.

- Initially, H-L pair is loaded with the address of first memory location.

- The first operand is moved to register B from memory location 0100H and H-L pair is incremented to point to next memory location.

- The second operand is moved to register C from memory location 0101H to act as the counter.

- The accumulator is initialized to 00H. Register B is added to the accumulator and the result is stored in the accumulator.

- Register C (counter) is decremented by 1. Then, the counter is checked for zero.

- If it hasn’t become zero yet, then register B is again added to the accumulator, and the counter is again checked for zero.

- If counter becomes zero, then H-L pair is incremented and the result is moved from the accumulator to memory location 0102H.

Flowchart:

Program:

LXI H, 0100H

MOV B,M

INX H

MOV C,M

MVI A, 00H

top: ADD B

DCR C

JNZ top

INX H

MOV M,A

HLT

Precautions:

- All steps should be followed carefully.

- Make sure all power sources are disconnected.

- Make sure you are properly grounded.

- Don’t touch the live wire

EXPERIMENT 1. (d): Division Of Two 8-Bit Numbers

Aim: Write a program using 8085 & Verify for: Division of Two 8-Bit Numbers

Apparatus: Microprocessor Kit 8085 , Key Board, Op-Code Sheet.

Theory:

- Load the HL pair registers with address of memory location.

- Move the first data to register B.

- Move the second data to the accumulator.

- Then compare two numbers for carry.

- Subtract the content of Register B to the content of accumulator.

- Then increment the value of carry.

- Then check whether the repeated subtraction is over.

- If the repeated subtraction is over then store the value of quotient and remainder in the given memory location.

Flowchart:

Program:

LXI H,3500H

MOV A,M

INX H

MOV B,M

MVI C,00H

LOOP: CMP B

JC skip

SUB B

INR C

JMP LOOP

skip: INX H

MOV M,C

STA 3503H

HLT

Precautions:

- All steps should be followed carefully.

- Make sure all power sources are disconnected.

- Make sure you are properly grounded.

- Don’t touch the live wire.

EXPERIMENT 2. (a): 1’s Complement of 8-Bit Number

Aim: Write a program for 8085 and verify it for: 1’s Complement of 8-Bit Number

Apparatus: Microprocessor Kit 8085 , Key Board, Op-Code Sheet.

Theory:

In this experiment to obtain 1’s compliment of a number its 0 bits are replaced by 1 and 1 by 0. The number is placed in the memory location 3000.The result is stored the memory location 3001

Flowchart:

Program:

LDA 3000H

CMA

STA 3001H

HLT

Precautions:

- All steps should be followed carefully.

- Make sure all power sources are disconnected.

- Make sure you are properly grounded.

- Don’t touch the live wire.

EXPERIMENT 2. (b): 2’s Complement of 8-Bit Number

Aim: Write a program for 8085 and verify it for: 2’s Complement of 8-Bit Number

Apparatus: Microprocessor Kit 8085 , Key Board, Op-Code Sheet.

Theory:

In this experiment to obtain 2’s compliment of a number is obtain by adding 1 to the 1’s compliment of the number. The number is placed in the memory location 3000. The result is to be stored in the memory location  3001.

Flowchart:

Program:

LDA 3000H

CMA

INR A

STA 3001H

HLT

Precautions:

- All steps should be followed carefully.

- Make sure all power sources are disconnected.

- Make sure you are properly grounded.

- Don’t touch the live wire.